Expderiment 3

object

To find out the viscosity of water by Poissellous Method.

apparatus

Glass jar, flask, graduated cylinder, capillary tube, stop watch, traveling microscope, meter rod, water tank, stand.

formula

η =    πρghr4     (poise)                    1 poise = 1dyne*sec/cm²

8LV

where ,

η = viscosity of glycerin (poise),

g = acceleration due gravity (981 cm/sec²),

ρ = density of water = 1 gm/cm3,

h = height of water inside flask = 16cm,

r = radius of capillary tube (cm) = 0.1325,

L = length of capillary tube (cm) = 26.2,

V = volume of water per second (m/sec).

A

B

C

D

Observation For Radius

Least Count = 0.001cm.

| S. No. | At | Main Scale Reading M.S.R.(cms) | Vernier Scale Reding V.S.R.(cms) | Fractional Part F.P.(cms) | Total Reading T.R.(cms) |
| --- | --- | --- | --- | --- | --- |
| 1 | A | 4.85 | 0.0 | 0.0 | 4.85 |
| 2 | B | 4.55 | 1 | 0.001 | 4.551 |
| 3 | C | 6.65 | 3 | 0.003 | 6.653 |
| 4 | D | 6.9 | 0.0 | 0.0 | 6.9 |

Observation For Viscosity of Water

| S. NO | VOLUME PER SECOND V= ml / sec |  |  | Height Of water Inside the Flask h (cm) | Viscosity Of Water  (poise) | Mean Viscosity Of water  (poise) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Volume (ml) | Time (sec) | Volume per second v / sec |  |  |  |
| 1. | 100 | 18.11 | 454 | 16 | 0.0152 | 0.0454158 |
| 2. | 200 | 38.65 | 5.1744545 | 16 | 0.0162 |  |
| 3. | 300 | 55.43 | 4545.412 | 16 | 0.0162 |  |

Calculation For Poise

For  V = 100ml.

η1= (3.142) (1.0 gm/cm3) (981 cm/sec2) (16 cm) (0.1375 cm)4

8 (26.2 cm) (5.521 v/sec)

η1 = 0.0152 (poise)

## For V = 200ml

η2= (3.142) (1.0 gm/cm3) (981 cm/sec2) (16 cm) (0.1375 cm)4

8 (26.2 cm) (5.174 v/sec)

η2 = 0.0162 (poise)

For V = 300ml

η3 = (3.142) (1.0 gm/cm3) (981 cm/sec2) (16 cm) (0.1375 cm)4

8 (26.2 cm) (5.412 v/sec)

η3 = 0.0162 (poise)

## MEAN POISE =  (η1 + η2+ η3) / 3

=  (0.0152 + 0.0162 + 0.0162) / 3

= 0.0158 poise

## η(poise)  =  0.0158 poise

Percentage Error = Original Value – Observed Value  x  100

Original Value

= 0.010 – 0.015  x  100

0.010

= 	-50		  OR

=      -0.5%

errors

- Due to disturbance of traveling microscope and stand radius can be measured wrong.

- The capillary tube may not be perfectly horizontally fitted in flask.

.

precautions

- Traveling microscope should handle with care.

- Make sure that it should not move during taking readings.

- .