Experiment-6

object

To determine the Young’s Modulus for a metal bar by bending (STATIC METHOD)

apparatus

Two knife edges, uniform metal bar, weight box, traveling microscope, vernier caliper.

formula

Y = gL3     X    M

4bd3	 X

Where

G = Accelaration due to gravity = 981 cm/sec

L = Length between two knife edges = 90 cm

b = Breadth of the rod = 1.2 cm

d = Thickness of the rod = 0.409 cm

M = Load on the pan = 200

X = Depression

Y = Coefficient of Rigidity

Table of observations

Least Count = 0.001cm

| S. No. | Load in gms | Microscope Reading |  |  | Depression for 200 gm cm | Mean |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Load Increasing cm | Load Decreasing cm | Mean |  |  |
| 1 | 0 | 7.40 | 7.40 | 7.40 | 0.23 | 0.21 |
| 2 | 200 | 7.15 | 7.17 | 7.17 | 0.14 |  |
| 3 | 400 | 7.00 | 7.00 | 7.00 | 0.20 |  |
| 4 | 600 | 6.80 | 6.80 | 6.80 | 0.20 |  |
| 5 | 800 | 6.60 | 6.60 | 6.60 | 0.25 |  |
| 6 | 1000 | 6.35 | 6.35 | 6.35 |  |  |

calculation

Y = gL3     X    M

4bd3	 X

Y =      (981)(90)3        X   200

4(1.2)(0.40)3           0.21

Y =    143029800000

0.064512

Y  =    2.217 X 1012 Dyne/cm

result

Young’s Modulus for a metal bar is found to be 2.217 X 1012 Dyne/cm2

Sources of error

- Edges supporting the bar may not be sharp.

- Position of the edges may have changed during the experiment.

- Inaccuracy in measurement of thickness of the bar.

- Inaccurate measurement of depression caused by the suspended load.

- Weight may not be accurate.

.

precautions

- Edges on which the bar is supported must be sharp and rigid and must be perpendicular to length of the bar.

- Load must be changed in regular steps.

- Load must be suspended at the center of gravity of the bar and it’s    distance must be equal from the to knife edges. For this purpose, bar of uniform thickness must be used.

- A slide error in the measure of “d” will cause considerable error in the final result.

- Position of the sharp edges must be kept fixed trough out the experiment.